# METHOD OF STATEMENT

Date: 24th July 2023

Description of work

1) To hoist up 5 unit machine to 2nd floor through swing gate opening by using platform and lorry crane

2) To hoist down 1 unit machine from 2nd floor through swing gate opening by using platform and lorry crane

3) To hoist down machine 4 unit from lorry chassis, shift and position to 2nd & 4th floor via cargo lift

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

11/07/23 (Fri) move in machine via cargo lift

12/08/23 (Sat) move in machine via hoisting

Time of Operation

10:00 am

Location of Operation

#### Blk 9 Kallang Place #02-06 & #04-11

Scape Precision Pte Ltd

Equipment Use

1 x 50 ton lorry crane

1 x 45 ton lorry crane

2-3 x lorry

1 x platform

Wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile/lorry crane at designated hoisting area for the lift

- Hoist up platform to 2nd floor swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on building opening slab

- Tied back platform to a competent point and secured using 1 x 45ton lorry crane

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- 1 x 50 ton mobile crane to hoist up machine to 2nd floor

- Lower down machine slowly onto platform

- Insert rollers/pallet truck to machine bed

- Shift out machine from platform

- Position machine into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175