# METHOD OF STATEMENT

Date: 27th Nov 2017

Description of Work

To unload 1 unit machine from lorry chassis, shift and position into customer premise at 3rd floor via cargo lift

Machine Spec

1 unit QTP-100

Dim: 1395 x 1285 x 1700 mm

Wt: 2000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

03/12/2017   –   09:30 am

Location of Operation

#### Blk 970 Toa Payoh North #03-01

#### Nortech Precision Pte Ltd

Equipment Use

1 x lorry crane

Pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated loading area for unloading of machine

- Hoist up machine from lorry chassis

- Insert pallet truck to machine base

- Shift and transport machine to 3rd floor via cargo lift

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175