# METHOD OF STATEMENT

Date: 31st Oct 2016

Description of work

1) To shift out and hoist down machineries from 2nd & 3rd floor by using platform and      mobile/lorry crane, load onto lorry chassis and deliver to new location

2)  Shift out small machine and accessories from 2nd & 3rd floor via cargo lift

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

18/11/2016   -   Shift out small machine and accessories via cargo lift

19/11/2016   -   Shift out and hoist down machine using platform and mobile/lorry crane

Location of Operation

#### Blk 998 Toa Payoh North #03-23/24/25/26

(Small Tool Technologies Pte Ltd)

Equipment Use

1 x 50 ton mobile

1 x 50 ton lorry crane

Lorry crane

1 x platform

Wire Sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade operation area, assignment of traffic controller to control movement within the vicinity                                                            All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness etc..) Lifting machine and lifting gear with M.O.M certificate

# JOB SCOPE

- Mobile/lorry crane to park at designated hoisting area for the lift

- Barricade operation area with safety cones and red/white tape

- Hoist up platform to 3rd floor opening

- Tied back platform to a competent point and secured using 50 ton lorry crane

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- Shift out machine from production room into platform

- 50 ton mobile crane to hoist down machine from platform

- Lower down machine and load onto lorry chassis

- Secured machine with belt prior to moving out of customer premise and deliver to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175