# METHOD OF STATEMENT

Date: 05th July 2016

Description of work

1) To unload 1 unit machine from lorry chassis by using loading bay, flush lorry to level with loading bay, cover gap with metal plate and to shift and position into customer premise at ground floor by using rollers/pallet truck

2) No hoisting required for this job

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

19/07/2016 – 09:30 am

Location of Operation

#### Blk 3015A Ubi Rd 1 #01-02

Macis Pte Ltd

Equipment Use

1 x lorry crane

Rollers/pallet truck

Crowbar/manual jack

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (etc.…safety shoes, helmet)

# JOB SCOPE

- Reverse lorry to loading bay

- Flush lorry to level with loading bay

- Cover gap with metal plate to roll machine out

- Jack up machine to 3’ high

- Insert rollers/pallet truck to machine base

- Shift out machine from lorry chassis and position into customer premise at ground floor

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris , if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175