# METHOD OF STATEMENT

Date: 07th Oct 022

Description of work

To unload 1 unit machine from lorry chassis by using loading bay, shift and position into customer premise at ground floor

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

19/10/22   -   02/11/22

Time of Operation

09:45 am

Location of Operation

#### Blk 16 Kallang Place #01-23

Rei Promax Pte Ltd

Equipment Use

2 x lorry loader

Rollers/pallet truck

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting operation are supervised by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of lorry loader with machine to location

- Reverse lorry to unloading bay

- Flush up lorry to level with loading bay

- Jack up machine

- Insert rollers under machine bed

- Shift out machine from lorry chassis

- Transport machine to tenant premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175