# METHOD OF STATEMENT

Date: 01st Sep 2020

Description of work

1) To hoist down and unload 2 unit machine from lorry chassis, shift out and position into customer premise at ground floor

2) To shift out 1 unit old machine from tenant premise, hoist up and load onto lorry chassis

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Est: 09-10/09/21

Time of Operation

09:40 am

Location of Operation

#### Blk 16 Kallang Place #01-23

Promax Pte Ltd

Equipment Use

2 x lorry crane

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gears with M.O.M certificate

Lifting operation are supervised by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Hoist down case from lorry chassis

- Uncrate case

- Hoist and detach machine from base

- Load machine onto lorry chassis

- Reverse lorry to unloading bay

- Flush up lorry to level with loading bay

- Insert rollers/pallet truck under machine base

- Shift out machine from lorry chassis

- Transport to tenant premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175