# METHOD OF STATEMENT

Date: 08th Jan 2018

Description of Work

To unload 1 unit machine with accessories from 1 x 20ft GP container, shift and position into customer premise

Machine Spec

Extruder JYZ 300-M Precision Preformer

Dim: 600 x 135 x 185 cm

Wt: 6, 700 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

10/01/2018   -   10:00 am

Location of Operation

#### No 10 Tuas South St 5

CDI Multiseals Pte Ltd

Equipment Use

1 x 45/50 ton lorry crane

Pallet truck/rollers

Wire/webbing sling

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated location (back to back with container)

- Crane to pull out machine from container onto lorry chassis

- Transport lorry to unloading area

- Hoist down machine from lorry chassis

- Lower down machine to near ground level

- Insert rollers to machine base

- Shift machine into premise and position to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175