# METHOD OF STATEMENT

Date: 13th July 2018

Description of work

To shift out 1 unit old pump from pump room at 3rd floor rooftop, hoist down to ground floor and to hoist up 1 unit new pump to 3rd floor rooftop, shift and position to pump room.

To relocate 1 unit pump and shift back to location after moving job is done

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

TBC

Location of Operation

#### Changi Complex

Equipment Use

1 x 50 ton mobile crane

1 x lorry crane

Rollers, pallet truck

Wire/webbing sling/shackle

Plywood/metal plate (to protect flooring from damage)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (etc.…safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Shift out old pump at 3rd floor pump room to hoisting area

- 50 ton mobile crane to hoist down old pump to ground floor

- Hoist up new pump to 3rd floor rooftop

- Transport pump into pump room

- Shift and position to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175