# METHOD OF STATEMENT

Date: 22 May 2015

Description of Work

To unload 1 unit Climbing Rock Stone using lorry crane, insert rollers/pallet truck, shift to playground area. Use of A Frame structure with 10 ton chain block to lower down stone into hole 30 cm deep

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

To be advised

Equipment Use

1 x lorry crane

Rollers/pallet truck

Wire/webbing sling

A Frame structure

Chain block

Obstacles

No obstacles on crane parking area and route to stone position

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

# JOB SCOPE

- Crane to hoist down climbing stone from lorry chassis

- Load machine onto the ground

- Insert rollers/pallet truck to base of stone

- Shift stone from loading area to designated location

- Set up  ‘A’ frame structure at location

- Using chain block attached to ‘A’ frame structure, to lift up stone slightly above ground

- Push structure together with stone to the exact location

- Release chain block slowly and let stone sit on the location inside hole

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175