# METHOD OF STATEMENT

Date: 09th Apr 2018

Description of work

To hoist up 3 unit machine to 2nd floor by using platform and mobile/lorry crane

Risk Assessment

A copy of Risk Assessment will be attach

Date/Time of Operation

14/04/2018   -   12:00 pm

Location of Operation

#### Blk 5000 AMK Ave 5 #02-05

Ding Feng Precision Engineering Pte Ltd

Equipment Use

1 x 50 ton mobile crane

2 x lorry crane

1 x platform

Wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness,  etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile/lorry crane to park at designated location for the hoist

- Hoist up platform to 2nd level swing gate opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 45 ton lorry crane

- Use of strong belt to secure front left and right of platform to prevent platform swing outward

- 50 ton mobile crane to hoist up machine to 2nd floor

- Lower down machine onto platform

- Insert pallet truck/rollers onto machine base

- Move out machine from platform

- Shift and position machine into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regard

Wei Long

Hup Hin Transport Co Pte Ltd

Tel: 62621525