# METHOD OF STATEMENT

Date: 24th Jan 2022

Description of Work

To assist engineer to dismantle machine column using gantry crane, load and secure onto I-Beam base, shift out and load onto Malaysia low bed

Machine Spec

TUE 10

Wt: 16, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

21/02/22 – set up gantry crane

22/02/22 – dismantle column and load onto I-Beam base

23/02/22 – shift out machine, column to loading bay and load onto Malaysia low bed

Time of operation

09:45 am

Location of Operation

51 Corporation Rd

Avitron Pte Ltd

Equipment Use

1 x 633 lorry loader – PALFINGER PK110002SHH

1 x forklift

Malaysia low bed

1 set hydraulic gantry crane

Rollers

Shackle/wire sling

Canvas/metal sheet (for floor protection)

Obstacles

To clear all obstruction in way of crane parking and working area

Barricade work area to prevent persons who are not involved enter unintentionally

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting gears with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Unload gantry crane from low bed

- Setting up of gantry crane structure at location

- Secure wire sling to column lifting point onto gantry crane

- Lift up column to required height

- Shift out gantry crane

- Lay down column and load onto I-Beam base

- Secure column onto base with bolt and nuts

- Shift out column and machine to loading bay

- Attached required lifting gears to crane hook and secured to rigging point of load

- Hoist up load to the required height

- Reverse Malaysia low bed to near machine

- Lower down and load machine onto low bed chassis

- Release all sling from machine hoisting point

- Secure machine with strapping belt prior to moving out of site premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175