# METHOD OF STATEMENT

Date: 10th Mar 2021

Description of Work

To assist engineer to dismantle 1 unit machine to 3 parts using gantry crane, after dismantle, shift and put aside

Machine Spec

1 unit Aida 600 Press machine – to be dismantle to 3 parts

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

23/03/21

Time of operation

09:45 am

Location of Operation

2 Benoi Crescent

SESS

Equipment Use

1 set hydraulic gantry crane

1 x 633 lorry loader

1 x 4.5 ton forklift

1 set heavy duty rollers

4 pcs 55ton shackle

4 pcs 20ton shackle

4 x 3m webbing sling

Obstacles

To clear all obstacles around working area

Barricade work area to prevent persons who are not involved enter unintentionally

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman will be involved in this operation

Safety Aspect

Barricade work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting gears with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of gantry crane structure at location

- Secure wire sling to machine lifting point onto gantry crane

- Lift up machine part to the required height

- Shift out gantry crane

- Lower down machine and put aside

- Repeat process till all machine parts have been dismantle

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175