# METHOD OF STATEMENT

Date: 26 June 2015

Description of Work

To unload 5 unit Vertical Milling machine from lorry chassis, uncrate, shift and position into customer premise

Risk Assessment

A copy of Risk Assessment will be attach.

Date / Time of Operation

To be advised

Location of Operation

#### No 500 Dover Rd – Singapore Polytechnic

W1412 School of Mechanical & Aeronautical Engineering

Equipment Use

1 x lorry crane

Pallet truck/rollers

Webbing sling/shackle

Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Hoist down cases from lorry chassis and load onto the ground

- Uncrate case

- Hoist and detach machine from base

- Insert pallet truck/rollers below machine base for the move

- Shift and position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175