# METHOD OF STATEMENT

Date: 18th July 2017

Description of work

To unload 3 unit High Speed Grinding machine with accessories from 1 x 20ft GP container, shift and position to 5th floor via cargo lift

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

19/07/2017

Location of Operation

#### No 18 Woodlands Loop

Edmund Optics Singapore Pte Ltd

Equipment Use

1 x lorry crane

Wire/webbing sling

Pallet truck/rollers

1 x forklift

Obstacles

Clear obstruction in way of working area and route to machine position

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (etc... safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

# Job Scope

- Lorry crane to park at designated location (back to back with container)

- Pull out machine from container and load onto lorry chassis

- Hoist down machine from lorry chassis to the ground

- Detach machine from base

- Insert pallet truck/rollers to machine base

- Transport machine to 5th floor via cargo lift

- Shift and position machine to designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175