# METHOD OF STATEMENT

Date: 21st Sep 2017

Description of work

To unload 1 unit machine from 1 x 20ft FR container, uncrate, hoist up to 5th floor by using platform and mobile/lorry crane

Machine Spec

1 unit STC machine

Dim: 191 x 191 x 220 cm

Wt: 6, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

23/09/2017   -    09:45 am

Location of Operation

#### 18 Woodlands Loop

Edmund Optics (S) Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 45 ton lorry crane

1 x platform

Wire Sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile/lorry crane to park at designated hoisting area for the lift

- Hoist up platform to 5th floor opening

- Tied back platform to a competent point and secured using 45 ton lorry crane

- Use strong belt to secured front left and right of platform to prevent platform swing outward

- 50 ton mobile crane to hoist up machine to 5th floor

- Lower down machine and load onto platform

- Insert rollers/pallet truck to machine base

- Shift out from platform and position into customer production room

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175