# METHOD OF STATEMENT

Date: 10th Aug 2021

Description of Work

To unload machine and parts from FR container using factory 38ton overhead crane, uncrate, shift and position to the designated location

Machine Spec

You Ji VTL-2500 CNC

Column: 5.45 x 4.05 x 2.82 m

Wt: 30, 000 kg

Table: 5 x 4.2 x 2.62 m

Wt: 30, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

17-20/08/21

Time of Operation

09:45 am

Location of Operation

#### 22 Jalan Terusan

NOV

Equipment Use

1 x 633 lorry loader

Factory 38ton overhead crane

Wire/webbing sling

Shackle

Obstacles

Clear obstruction in way of working area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Reverse FR container into inside factory

- Attached lifting gears onto overhead crane hook and secured to load to be lifted

- Hoist down case from container and load onto the ground

- Uncrate case

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to the ground

- Assist customer engineer to install machine parts

- After all parts are install, position machine to the designated location

- Unload machine and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175