# METHOD OF STATEMENT

Date: 23rd Oct 2025

Description of work

To reverse 4 x 40ft OT container into inside factory premise, unload item from container using customer overhead crane, shift and position to designated location

Machine Spec

31 packages of 1 unit Vanguard Wellhead CNC Lathe

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate:

25/10/25 – 31/10/25

Time of Operation

09:45 am

Location of Operation

#### 80 Tuas West Dr

Innovex Asia Pacific Pte Ltd

Equipment Use

2 x 633 lorry loader

Customer 20ton overhead crane

1 x scissor lift

Wire sling

Webbing sling

Shackle

Obstacles

Clear obstruction in way of working area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M qualified lifting supervisor, rigger and signalman

# Job Scope

- Reverse container into inside factory premise

- Remove up container canvas

- Rigger to insert sling onto overhead crane hook

- Secure sling to rigging point of machine

- Using customer 20ton overhead crane to hoist up machine from container

- Swing crane and lower down machine to near ground level

- Shift and position to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175