# METHOD OF STATEMENT

Date: 10th Jan 2023

Description of work

To shift out following equipment from production to loading area, load up onto lorry chassis and deliver to new location

Equipment Spec

3 unit Hydraulic Test Stand

Dim: 300 x 100 x 130 cm

Wt: 800 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

12/01/23

Time of Operation

Afternoon schedule

Location of Operation

#### Flowserve Pte Ltd

#### Move out: 12 Tuas Ave 20

Move in: 7 Tuas Loop

Equipment Use

1 x lorry loader

Pallet truck

Crowbar

Wire / webbing sling

Obstacles

No obstacle on crane parking area and route of moving operation

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involve in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lift up equipment using crowbar

- Insert pallet truck onto equipment base

- Shift out from production to loading bay

- Lorry loader to hoist up and load equipment onto lorry chassis

- Secured equipment with strapping belt prior to moving out of site premise

- Transport and deliver equipment to new factory

- At new factory, unload down equipment from lorry chassis

- Shift and transport into factory premise

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175