# METHOD OF STATEMENT

Date: 17th Nov 2022

Description of work

To shift out following equipment from production to loading area, fork up onto lorry chassis and deliver to new factory

Equipment Spec

1 unit Pressure Tester

1 unit Think PC Progetti

1 unit computer cabinet

1 unit water tank

Risk Assessment

A copy of Risk Assessment will be attach

Schedule of Operation

23/11/22

Time of Operation

09:45 am

Location of Operation

#### Flowserve Pte Ltd

#### Move out: 12 Tuas Ave 20

Move in: 7 Tuas Loop

Equipment Use

1-2 x lorry loader

1 x forklift

Pallet truck

Crowbar

Obstacles

No obstacle on crane parking area and route of moving operation

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Safety Aspect

Barricade off work area with safety cones, red/white tape

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified forklift operator

# JOB SCOPE

- Insert pallet truck onto equipment base

- Shift out from production to loading bay

- Using forklift to load upequipment onto lorry chassis

- Secured equipment with strapping belt prior to moving out of site premise

- Transport and deliver to new factory

- At new factory, unload down equipment from lorry chassis

- Shift and transport into factory premise

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175