# METHOD OF STATEMENT

Date: 20th Mar 2023

Description of work

To shift out following machine and parts from production to loading area, hoist up onto lorry chassis and deliver to new factory

Machine Spec

1 unit Amada 8025 Banding / Dim: 280 x 150 x 240 cm / Wt: 5, 400 kg

1 unit Amada 1030 Bending / Dim: 330 x 170 x 260 cm / Wt: 7, 000 kg

1 unit Murata 2000R Turret Punch / Dim: 360 x 280 x 200 cm / Wt: 7, 000 kg

1 unit LVD Shearing machine / Dim: 410 x 160 x 160 cm / Wt: 5, 000 kg

1 unit Oven / Dim: 410 x 300 x 244 cm / Wt: 3, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Schedule of Operation

23/03/23

Time of Operation

09:45 am

Location of Operation

#### Philip Electric Pte Ltd

#### Move out: 17 Defu Lane 7

Move in: 56 Loyang Way

Equipment Use

2 x 633 lorry loader

1 x low bed

Rollers/pallet truck

Hand jack/crowbar

Wire/webbing sling

Shackle

Woodblock

Obstacles

No obstacle on crane parking area and route of moving operation

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involve in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine to 3” high

- Insert rollers/pallet truck under machine bed

- Shift out machine from production to hoisting bay

- Attached required lifting gears to crane hook and secured to rigging point of machine

- Lorry loader to hoist up machine and load machine onto low loader/lorry chassis

- Release all sling from machine hoisting point

- Secured machine with strapping belt prior to moving out of customer premise

- Transport and deliver machine to new factory

- At new factory, hoist down machine from low loader/lorry chassis

- Insert rollers/pallet truck under machine bed

- Shift and transport machine into factory premise

- Position machine at the designated location

- Unload machine from rollers/pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175