# METHOD OF STATEMENT

Date: 16th Aug 2024

Description of work

To shift out following machine and parts from production to loading area, load up onto lorry chassis using forklift and lorry crane

Machine Spec

1 unit Folding machine

4 unit Ironer machine

3 unit Washing machine

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

21/08/24

Time of Operation

09:45 am

Location of Operation

#### Prestige Laundry & Cleaning Pte Ltd

#### Move out: 79 Pandan Loop

Move in: 99 Pandan Loop

Equipment Use

2 x 633 lorry loader

1 x 7ton forklift

Rollers

Pallet truck

Hand jack

Crowbar

Woodblock

Wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area and route of moving operation

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involve in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine to 3” high

- Insert rollers/pallet truck under machine bed

- Shift out machine from production to loading area

- Rigger to attached sling to crane hook

- Secured sling to rigging point of load to be lifted

- Lorry loader to hoist up machine and load onto lorry chassis

- Release all sling from machine hoisting point

- Secured machine with strapping and transport to new factory

- Using of 7ton forklift to fork up machine

- Load up machine onto lorry chassis

- Secured machine with strapping belt

- Transport and deliver machine to new factory

- At new factory, unload machine from lorry chassis to loading area using forklift and lorry loader

- Insert rollers/pallet truck under machine bed

- Shift and transport machine into factory premise

- Position machine at the designated location

- Unload machine from rollers/pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175