# METHOD OF STATEMENT

Date: 27th Aug 2025

Description of work

To shift out following machine and parts from ground floor @ Blk 4009 to blk 4008 loading bay, hoist up onto lorry chassis and deliver to new factory

Machine Spec

Refer to attached list

Risk Assessment

A copy of Risk Assessment will be attached

Schedule of Operation

Estimate 3 to 5 days job

09/09/25 – 13/09/25

Time of Operation

09:45 am

Location of Operation

#### Royal Insignia Pte Ltd

#### Blk 4009 A.M.K Ave 10 #01-37

#### Blk 4008 A.M.K Ave 10 #01-19

Equipment Use

2-3 x lorry loader

Rollers/pallet truck

Crowbar / woodblock

Wire / webbing sling

Shackle

Obstacles

No obstacle on crane parking area and route of moving operation

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involve in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lift up machine to 3” high

- Insert rollers/pallet truck under machine bed

- Shift out machine from production to loading bay

- Rigger to attached required lifting gears to crane hook and secured to rigging point of load to be lifted

- Lorry loader to hoist up machine

- Load machine onto lorry chassis

- Secured machine with strapping belt prior to moving out of customer premise

- Transport and deliver machine to new factory

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175