# METHOD OF STATEMENT

Date: 24th Feb 2023

Description of work

To unload 24 cases of machines and accessories from 1 x 40ft FR and 2 x 40ft HQ container, uncrate, shift and position into factory premise at ground floor

Machine Spec

Refer to attached packing list

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate:

18-21/09/23

Time of operation

09:45 am

Location of Operation

#### 273 Jalan Ahmad Ibrahim

Baker Hughes Singapore Pte

Equipment Use

1 x 633 lorry loader

1 x 3ton forklift

1 x 10ton forklift

Rollers/pallet truck

Metal plate/canvas (to lay out for floor protection)

Obstacles

No obstacle around operation area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified forklift operator will be involved in the operation

# JOB SCOPE

- Using 4.5ton forklift to fork down case from FR container to the ground

- Uncrate case and put aside

- Forklift to fork up and detach machine from base

- Transport machine to loading bay

- Lower down machine to near ground level

- Insert roller/pallet truck under machine base

- Shift and transport into production area

- Position to the designated location

- Unload from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175