# METHOD OF STATEMENT

Date: 13th Sep 2023

Description of work

To unload machine and accessories from 1 x 40ft FR and 2 x 40ft HQ container using forklift, uncrate, shift and position into factory premise at ground floor

Machine Spec

Refer to packing list attached

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate:

18-21/09/23

Time of operation

09:45 am

Location of Operation

#### 273 Jalan Ahmad Ibrahim

Baker Hughes Singapore Pte

Equipment Use

1-2 x 633 lorry loader

1 x 10ton forklift

1 x 3ton forklift

Rollers/pallet truck

Hand jack/woodblock

Metal plate/canvas (to lay out for floor protection)

Obstacles

No obstacle around operation area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Barricade off work area with safety cones, red/white tape and signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

M.O.M certified lifting supervisor, rigger, signalman, lorry loader and forklift operator will be involved in the operation

# JOB SCOPE

- Using 10ton forklift to fork down case from FR container to the ground

- Uncrate case and put aside

- Forklift to fork up and detach machine from base

- Transport machine to loading bay

- Lower down machine to near ground level

- Insert roller/pallet truck under machine base

- Shift and transport into production area

- Position to the designated location

- Unload from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175