# METHOD OF STATEMENT

Date: 03rd June 2021

Description of Work

To unload 2 cases of 1 unit machine from lorry chassis using forklift, uncrate, shift and transport to 4th floor via cargo lift

Machine Spec

2D Optical Vision Measuring machine

Case 1: 137 x 93 x 133 cm / Wt: 350 kg

Case 2: 115 x 90 x 100 cm / Wt: 95 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

07/06/21

Time of Operation

09:30 am

Location of Operation

#### 6 Serangoon North Ave 5 #04-06/07/08

Identiv Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

1 x forklift

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified forklift operator

# Job scope

- Deploy lorry loader at designated hoisting area

- Using forklift to unload case from lorry chassis to the ground

- Uncrate machine case

- Fork up and detach machine from base

- Transport and unload at loading bay

- Using pallet truck to shift and transport machine to 4th floor via cargo lift

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175