# METHOD OF STATEMENT

Date: 26th Mar 2018

Description of work

To shift out 1 unit machine from production room @ 11 Tuas South Ave 12, hoist up onto lorry chassis and deliver to 37 Joo Koon Rd, unload, shift and position into premise at ground floor

Machine Spec

1 unit Kingstone HL300 Manual Lathe

Dim: 310 x 80 x 80 cm

Est Wt: 3, 500 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

28/03/2018   –   13:00 pm

Location of Operation

#### From - 11 Tuas South Ave 12 (Halliburton)

To - 37 Joo Koon Rd (Energy Pipeline Services Pte Ltd

Equipment Use

1 x lorry crane

Rollers/pallet truck

Wire/webbing sling

Metal sheet/canvas (for epoxy floor protection)

Obstacles

No obstacle on crane parking area and route to machine location

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lift up machine to 3” high

- Insert rollers to machine base

- Shift out machine from production room to hoisting bay

- Hoist up and load machine onto lorry chassis

- Secured machine with strapping belt prior to moving out of premise

- Transport and deliver  machine to new location

- At new premise, hoist down machines from lorry chassis

- Insert rollers to machine base

- Shift and transport machine into premise

- Position machine to the designated location

- Unload machine from rollers and let it sit on to machine plate

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175