# METHOD OF STATEMENT

Date: 10th Jan 2020

Description of work

To shift out following machine from production room to loading area, hoist up onto trailer and deliver to new location

Machine Spec

1 unit Fuji Seidakusho Deep Hole Drilling – to dismantle into 3 parts

Dim: 750 x 120 x 150 cm / Wt: 9, 000 kg

Dim: 580 x 120 x 150 cm / Wt: 6, 000 kg

Dim: 550 x 120 x 150 cm / Wt: 6, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time Of Operation

15/01/20   -   10:00 am

Location of Operation

#### Collection – 16 Gul Drive (Drilmaco Pte Ltd)

#### Delivery – 4 Tuas Ave 13 (IEG Oilfield Services Pte Ltd)

Equipment Use

1-2 x 633 lorry loader

2 x trailer

1 x forklift

Rollers/pallet truck

Hydraulic/Hand jack

1 set wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking are

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involve in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine to 4” high

- Insert rollers under machine base

- Shift out machine from production to hoisting area

- Using lorry loader to hoist up machine

- Load machine onto trailer

- Secured machine with belt prior to moving out of premise

- Transport and deliver machine to new location

- At new location, lorry loader to unload and hoist down machine from trailer

- Lower down machine to near ground level

- Insert rollers under machine bed

- Shift and transport machine into production

- Position and assemble machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175