# METHOD OF STATEMENT

Date: 21st Oct 2019

Description of work

To shift out following machine from production room to loading area, hoist up onto low bed/lorry chassis and deliver to new factory

Machine Spec

Refer to attached packing list

Risk Assessment

A copy of Risk Assessment will be attach

Schedule Of Operation

Est: 26/10/2019   -   26/11/2019

Time Of Operation

09:45 am

Location of Operation

#### Drilmaco Pte Ltd

#### Collection – 16 Gul Drive

#### Delivery – 11 Tuas Ave 6

Equipment Use

1 x 50/100 ton mobile crane

633 lorry loader

Low bed

Rollers/pallet truck

Hydraulic/Hand jack

1 set wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involve in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine to 4” high

- Insert rollers under machine base

- Shift out machine from production room to hoisting area

- Mobile/lorry crane to hoist up machine

- Load machine onto low bed/lorry chassis

- Secured machine with belt prior to moving out of customer premise

- Transport and deliver machine to new factory

- At new factory, unload machine from low bed/lorry chassis

- Shift machine into premise

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175