# METHOD OF STATEMENT

Date: 28th June 2018

Description of work

To shift out 1 unit machine and accessories from production @ 23 Loyang Way, hoist up onto low-loader and deliver to 6 Seletar Aerospace Link, unload, shift and position into premise at ground floor

Machine Spec

1 unit DMG EMU65 MonoBlock CNC Milling

Dim: 340 x 250 x 310 cm

Wt: 11, 500 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

TBA

Location of Operation

#### From 23 Loyang Way – 6 Seletar Aerospace Link

GE Aviation

Equipment Use

1 x 50 ton mobile/lorry crane

1 x lorry crane

1 x low-loader

Rollers/pallet truck

Wire/webbing sling

Canvas/metal sheet (for epoxy floor protection)

1 x forklift

Obstacles

No obstacle on crane parking area and route to machine location

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine

- Insert rollers to machine base

- Shift out machine from production room to hoisting bay

- Hoist up and load machine onto low-loader

- Secured machine with strapping belt prior to moving out of premise and deliver to new location

- At new premise, hoist down machines from low-loader

- Insert rollers to machine base

- Transport machine into production area

- Shift and position machine to the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175