# METHOD OF STATEMENT

Date: 11th Dec 2017

Description of work

To shift out 2 unit machine from production @ 8 Tuas West Ave, hoist up onto lorry/low-loader and transport to 34 Benoi Rd, unload, shift and position into premise at ground floor

Machine Spec

1 unit Danichi MX105 CNC Milling

Dim: 800 x 200 x 230 cm

Wt: 15, 000 kgs

1 unit Haas TM3 CNC Milling

Dim: 300 x 200 x 230 cm

Wt: 3, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

13/12/2017   –   10:00 am

Location of Operation

#### From No 8 Tuas West Ave – No 34 Benoi Rd

(Hunting Energy Services Pte Ltd)

Equipment Use

1 x 75 ton lorry crane

1 x 50 ton lorry crane

Lorry/low loader

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area and route to machine location

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine

- Insert rollers to machine base

- Shift out machine from production room to hoisting bay

- Hoist up and load machine onto low-loader/lorry

- Secured machine with strapping belt prior to moving out of premise to new location

- At new premise, hoist down machines from low-loader/lorry

- Insert rollers to machine base

- Shift and position machine to the designated location inside factory

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175