# METHOD OF STATEMENT

Date: 06th Nov 2017

Description of work

To unload 2 cases of 1 unit Mazak Horizontal NC Lathe machine from 1 x 20ft FR and 1 x 20ft GP container, uncrate, shift and position into customer premise at ground floor

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

13/11/2017   -   09:30 am

Location of Operation

#### Blk 5004 A.M.K Ave 5 #01-10

GS Precision Pte Ltd

Equipment Use

1 x 50 ton lorry crane

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated location for the hoist

- Hoist down case from FR container onto ground

- Uncrate case

- Hoist up and detach machine from base

- Load machine onto lorry chassis

- Reverse lorry to loading bay

- Flush up lorry crane to level with loading bay

- Shift out machine from lorry chassis and position into customer premise at ground floor

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regard

Wei Long

Hup Hin Transport Co Pte Ltd

Tel: 62621525