# METHOD OF STATEMENT

Date: 04th Dec 2018

Description of Work

Install gantry crane to lift up machine column to 1ft high for repair, after repairing position back to location

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

07-10/12/2018   -   09:30 am

Location of Operation

51 Seletar Aerospace View

Pratt & Whitney NGPF Manufacturing Company

Equipment Use

2 x lorry crane

1 x low loader

1 set gantry crane

Wire sling/shackle

10 ton chain block

2 x forklift

Obstacles

To clear all obstacles around working area for easy access to location

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, lorry loader operator and forklift operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Unload gantry crane from low loader

- Shift into production area

- Setting up of gantry structure at location

- Slug sling onto column lifting point and secured onto gantry crane

- Lift up column to the required height

- Standby/insert wooden block onto column bed to stabilize it

- Engineer to do repairing work on the column

- After repairing, install back onto machine

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175