# METHOD OF STATEMENT

Date: 08th Sep 2021

Description of Work

To lift up machine column to 1ft high by using gantry crane for engineer to repair, after repairing position back to location

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

14/09/21   –   Deliver and set up gantry on site

15/09/21   -   Lift up column for engineer to do repairing job

Time of Operation

14/09/21   -   Afternoon schedule

15/09/21   -   9:45 am

Location of Operation

51 Seletar Aerospace View

Pratt & Whitney NGPF Manufacturing Company

Equipment Use

1 x lorry crane

1 x low loader

1 set gantry crane

Wire sling/shackle

1 x 6ton forklift

1 x 3ton battery forklift

Obstacles

To clear all obstacles around working area for easy access to machine location

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader and forklift operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Unload gantry crane from low loader

- Shift into production area

- Set up of gantry structure at location

- Insert sling onto column lifting point and secured onto gantry crane

- Lift up column to the required height

- Insert wooden block onto column bed to stabilize it

- Engineer to do repairing work on the column

- After repairing, install back onto machine

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175