# METHOD OF STATEMENT

Date: 19th Sep 2017

Description of work

To hoist up 4 unit machine to 2nd floor by using platform and mobile/lorry crane

Machine Spec

Kawashima Four-Side Horizontal Form Seal Machine

Case 1: 370 x 151 x 190 cm / Wt: 2, 470 kgs

Case 2: 307 x 166 x 214 cm / Wt: 2, 010 kgs

Case 3: 393 x 96 x 122 cm  /  Wt: 490 kgs

Case 4: 340 x 110 x 141 cm / Wt: 850 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

21/09/2017   -   10:00 am

Location of Operation

#### 2 Chia Ping Rd

Haw Par Healthcare Ltd

Equipment Use

1 x 50 ton mobile/lorry crane

1 x lorry crane

1 x platform

Scissor lifter

Wire/webbing sling

Metal plate (for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (etc…safety shoes, helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Mobile/lorry crane to park at designated location for the hoist

- Load machine onto platform

- Strap and secured machine onto platform with strapping belt

- Hoist up platform together with machine to 2nd level

- Stabilize platform effectively and ensure it is fully rested on the parapet wall

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- Insert pallet truck to machine base

- Shift out machine from platform and load onto scissor lifter

- Lower down scissor lifter, roll out machine and let it sit onto pallet truck/rollers

- Shift and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175