# METHOD OF STATEMENT

Date: 21st Dec 2017

Description of Work

1) To shift out 2 unit machine from tool room to loading area, hoist up onto lorry chassis, secured with strapping belt, transport to new location, unload, shift and position into customer premise

2) To relocate 2 unit Roeders RXP 400 machine within premise @ Blk G, Room G-L2

Machine Spec

2 unit Roeders RXP 300

Dim: 240 x 170 x 205 cm

Wt: 3, 000 kgs each

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

Estimate: 02-05/01/2018   -   09:30 am

Location of Operation

#### No 2 A.M.K Drive - ITE College Central

From Blk H, Room H-L2   -   Blk G, Room G-L2

Equipment Use

1 x 35/45 ton lorry crane

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off operation area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated hoisting area

- Shift out machine from tool room to loading area

- Hoist up machine and load onto lorry chassis

- Secured machine with strapping belt

- Transport machine to new location

- At new location, hoist down machine from lorry chassis

- Unload machine at designated loading area

- Insert rollers onto machine base

- Shift machine into premise

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175