# METHOD OF STATEMENT

Date: 02nd Aug 2022

Description of work

To relocate 8 unit machine within plant area

Machine Spec

2 unit Index E42 Auto Lathe

1 unit Paragon Conventional Grinding

1 unit Hwa Cheon Hi-Tech 200 CNC Lathe

1 unit Hamatsu Conventional Turning

1 unit Dryer

1 unit Walter Tumbling

1 unit Tumbling

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

19/08/22 (Friday)

Time of Operation

09:45 am

Location of Operation

#### 249 Jalan Boon Lay

ST Kinetics Singapore

Equipment Use

1 x 633 lorry loader

Crowbar/hand jack

Woodblock

Rollers/pallet truck

Obstacles

No obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

# Job Scope

# Lift up machine to 3” high

# Insert rollers/pallet truck under machine base

# Shift out machine from existing location

# Transport to new location within plant

# Position machine at designated location

# Unload from pallet truck and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175