# METHOD OF STATEMENT

Date: 11th Dec 2024

Description of work

To relocate 1 unit machine from existing location to bay 6

Machine Spec

1 unit Deep Hole Drilling

Wt: 8, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

26/12/24

Time of Operation

09:45 am

Location of Operation

#### 273 Jalan Ahmad Ibrahim

Baker Hughes (S) Pte

Equipment Use

1 x 633 lorry loader

1 x trailer

Hydraulic/hand jack

Woodblock

Rollers/pallet truck

Canvas/metal plate (for epoxy floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety glass)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

Job Scope

# Lift up machine using hydraulic/hand jack

# Insert rollers under machine base

# Shift out machine from existing location to loading area

# Riggers to insert sling to crane hook a

# Secured sling to rigging point of machine

# Lorry crane to hoist up machine and load up onto trailer chassis

# Transfer machine to bay 6

# Lorry crane to hoist down machine from trailer

# Lower down machine to near ground level

# Insert rollers under machine bed

# Shift and transport machine into factory premise

# Position machine at the designated location

# Unload machine from rollers and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175