# METHOD OF STATEMENT

Date: 01st Sep 2025

Description of work

To relocate 1 unit machine from existing location to new location about 10m away

To shift out 1 unit machine to loading bay, hoist up onto low-bed, transfer to another location within customer factory, unload, shift and position into factory premise

Machine Spec

Relocate:

1 unit Mazak QTN450 machine

Shift out to another location:

1 unit Mazak Cybertech Turn 4500T

Dim: Wt: 21, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

02/09/25

Time of Operation

09:30 am

Location of Operation

#### 273 Jalan Ahmad Ibrahim

Baker Hughes (S) Pte

Equipment Use

1 x 633 lorry loader

1 x 100ton mobile crane

1 x low-bed

Hydraulic/hand jack

Woodblock

Electric roller/rollers

Canvas/metal plate (for epoxy floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine new location

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety glass)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

Job Scope

# Lift up machine using hydraulic/hand jack

# Insert rollers under machine bed

# Shift out machine from existing location to loading area

# Riggers to insert sling to crane hook

# Secured sling to rigging point of machine

# Mobile crane to hoist up machine and load up onto low-bed

# Transfer machine to another location within factory premise

# Mobile crane to hoist down machine from low-bed

# Lower down machine to near ground level

# Insert rollers under machine bed

# Shift and transport machine into factory premise

# Position machine at the designated location

# Unload machine from rollers and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175