# METHOD OF STATEMENT

Date: 18th Jan 2018

Description of Work

To unload 1 unit machine from 1 x 40ft HQ container, uncrate, shift and position into customer CMM room

Machine Spec

Global Performance 12.22.10 CMM Machine

Packing Dim: 3928 x 2178 x 2549

Gross Wt: 6834 kgs   /   Net Wt: 5879 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

22/01/2018   -   09:40 am

Location of Operation

#### 16 Tampines Industrial Crescent

Jabil Circuit (S) Pte Ltd

Equipment Use

1 x 35/45 ton lorry crane

Pallet truck/rollers

Wire/webbing sling

Canvas/metal plate (for epoxy floor protection)

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated location (back to back with container)

- Crane to pull out machine case from container onto lorry chassis

- Hoist down case to the ground

- Uncrate machine case

- Hoist up machine from base and load back onto lorry chassis

- Transport lorry to loading/unloading area

- Insert pallet truck/rollers to machine base

- Move out machine from loading bay

- Shift and position into customer CMM room

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175