# METHOD OF STATEMENT

Date: 02nd June 2016

Description of Work

To unload 1 unit machine from low-bed, shift and position into customer premise at ground floor

Machine Spec

1 x Lathe Machine

Dim: 600 x 250 x 244 cm

Est Wt: 9,000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

07/06/2016  -  10:30 am

Location of Operation

NOV- PASIR GUDANG

Equipment Use

1 x lorry crane

Heavy duty rollers/pallet truck

Wire sling

1 x forklift

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

# JOB SCOPE

- Mobile crane to park at designated hoisting area for the lift

- Reverse low-bed near crane

- Hoist up machine from low-bed and lower down to ground level

- Insert rollers to machine base for the move

- Using forklift to assist in pushing/pulling machine into customer premise at ground floor

- After machine position at the designated location, jack up machine, remove rollers and let it sit on machine plate

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete