# METHOD OF STATEMENT

Date: 08th July 2019

Description of work

To unload 6 packages of 2 set Modula Manual Lifter from 1 x 40ft and 1 x 20ft container, shift and position into production at ground floor

Modula Lifter Spec

Refer to attached packing list

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

09/07/2019   -   09:30 am

Location of Operation

#### 16 Seletar Aerospace Crescent

JEP Precision Engineering Pte Ltd

Equipment Use

1 x lorry crane

1 x forklift

Wire/webbing sling

Canvas/metal plate (for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry crane and forklift operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervised by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- Lorry crane to park at designated location (back to back with container)

- Pull out skid from container onto lorry chassis

- Forklift to unload case from lorry chassis

- Using forklift/lorry crane to detach Modula Lifter from base

- Shift and position at designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175