# METHOD OF STATEMENT

Date: 22nd Oct 2020

Description of Work

To unload 2 case of 1 unit machine from 1 x 20ft OT container, uncrate, shift and position into factory premise at ground floor

Machine Spec

E-Tech Grinder EDG-1000 NC

Case 1: 242 x 195 x 297 cm / Wt: 5,000 kg

Case 2: 333 x 224 x 145 cm / Wt: 1,000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

27/10/2020 (Tuesday)

Time of Operation

09:30 am

Location of Operation

#### 16 Seletar Aerospace Crescent

JEP Precision Engineering Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x 10ton forklift

Rollers/pallet truck

Wire/webbing sling

Crowbar/hand jack

Canvas/metal plate (for epoxy floor protection)

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry crane operator and forklift operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated location for the lift

- Open up container canvas

- Secured sling under machine case

- Hoist up case from container and load onto the ground

- Uncrate case

- Using 10ton forklift to fork up and detach machine from base

- Transport and unload machine at door entrance

- Using pallet truck/rollers to shift machine into factory premise

- Position machine at the designated location

- Unload machine from pallet truck/rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175