# METHOD OF STATEMENT

Date: 28th May 2020

Description of work

To unload 4 unit machine at loading bay, shift out from lorry chassis and position into customer premise at ground floor

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

02-03/06/2020   -   09:30 am

Location of Operation

#### Blk 4 Kaki Bukit Ave 1 #01-07

Micro-Mechanics Pte Ltd

Equipment Use

2 x 633 lorry loader

1 set wire/webbing sling

Rollers

Pallet truck

Obstacles

No obstacle on crane and unloading area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- Lorry crane to park at designated location

- Unload case from lorry chassis to the ground

- Uncrate machine case

- Hoist up machine from base and load back onto lorry chassis

- Reverse lorry to unloading bay

- Insert rollers under machine base

- Shift out machine from lorry chassis

- Transport machine into tenant premise at ground floor

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175