# METHOD OF STATEMENT

Date: 01st Feb 2018

Description of Work

To unload 9 packages of 1 set Anaerobic Fermenter Reactor @ total weight of 1150 kgs from lorry chassis, uncrate, shift and position into factory

To tilt up 2000L Digester inside factory and position to location

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

07/02/2018   -   10:00 am

Location of Operation

#### 12 Science Park Drive

Equipment Use

1 x lorry crane

Pallet truck

Wire/webbing sling

1 x ‘A’ frame structure with 2/3 ton chain block

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# Job scope

- Lorry crane to park at designated hoisting area

- Hoist down packages from lorry chassis

- Uncrate equipment case

- Hoist up and detach equipment from base

- Insert pallet truck to equipment base for the move

- Shift and position equipment to location

- Using ‘A’ frame structure and chain block to tilt up 2000L Digester

- Shift and position to designated location

- To lift up 1 unit horizontal motor onto digester top

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175