# METHOD OF STATEMENT

Date: 23rd May 2019

Description of Work

To unload 1 unit machine and ‘A’ frame set from lorry chassis, shift and position into factory

Assemble ‘A’ frame and chain block to upright machine

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

23/05/2019   -   09:45 am

Location of Operation

#### 9 Winstedt Rd

LASALLE

Equipment Use

1 x lorry crane

Pallet truck

Wire/webbing sling

1 x ‘A’ frame structure with 3 ton chain block

1 x forklift

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Deploy lorry crane at designated unloading area

- Unload equipment from lorry chassis

- Insert pallet truck for the move

- Shift and position equipment to location

- Assemble ‘A’ frame structure and chain block

- Upright machine using ‘A’ frame and chain block

- Position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175