# METHOD OF STATEMENT

Date: 29th Nov 2018

Description of Work

To unload equipment from Malaysia lorry using lorry crane, shift and position into premise at ground floor

Equipment Spec

1 unit Rotary Table

Dim: 190 x 190 x 100 cm

Wt: 1, 700 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

10/12/2018   -   Afternoon schedule

Location of Operation

#### 3 Clean Tech Loop, Clean Tech 2 #01-01

STAR Advanced Remanufacturing & Technology

Equipment Use

1 x lorry crane

Pallet truck

Wire/webbing sling

Obstacles

No obstacles at unloading area and route to machine position

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Lorry crane to park at designated unloading area

- Hoist down machine from lorry chassis

- Detach equipment from base

- Swing crane away from base

- Insert pallet truck to equipment base

- Shift and transport equipment into premise at ground floor

- Position equipment at the designated location

- Drill and bolt down equipment

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175