# METHOD OF STATEMENT

Date: 09th Oct 2020

Description of Work

To unload 1 unit machine at unloading bay using lorry crane, shift and position into tenant premise at ground floor

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

15/10/2020 (Thursday)

Time of Operation

10:00 am

Location of Operation

#### Blk 1002 Jalan Bukit Merah #01-13

CNC Precision Pte Ltd

Equipment Use

1 x lorry crane

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade operation area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Using lorry crane to lift up machine and detach from base

- Reverse lorry crane to loading bay

- Flush up lorry jack to level with loading bay

- Insert rollers under machine base

- Shift out machine from lorry chassis

- Transport machine into tenant premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175