# METHOD OF STATEMENT

Date: 23rd Sep 2020

Description of Work

1) To unload 2 unit machine from container at unloading bay using lorry crane and forklift, uncrate, shift and position to location at ground floor

2) To shift out 2 unit machine from tenant premise at ground floor, load onto lorry chassis and deliver to another location

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

28/09/2020 (Monday)

Time of Operation

09:30 am

Location of Operation

#### Blk 16 Kallang Place #01-22/23

Rei Promax Technologies Pte Ltd

Equipment Use

1 x lorry crane

Pallet truck

Wire/webbing sling

1 x forklift

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade operation area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Deploy lorry loader at designated unloading area (back to back with container)

- Use crane to pull out case from container onto lorry chassis

- Forklift to fork down case from lorry chassis

- Uncrate case

- Use forklift to detach machine from base

- Unload machine at loading bay

- Insert pallet truck to machine base

- Transport machine into tenant premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175