# METHOD OF STATEMENT

Date: 20th Mar 2019

Description of Work

To unload machines and accessories from lorry chassis using lorry crane, shift and position into tenant premise at ground floor

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

29-30/03/2019

Time of Operation

12:00 pm

Location of Operation

#### Blk 7 Kallang Place #01-04

Global Advanced-Tech (S) Pte Ltd

Equipment Use

3-4 x lorry crane

Pallet truck/rollers

1 set wire/webbing sling

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crew certificate

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade operation area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Lorry crane to park at designated unloading area

- Hoist down machine from lorry chassis

- Swing crane to unloading area

- Lower down machine to near ground level

- Insert pallet truck/rollers to machine base

- Shift and transport machine into tenant premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175