# METHOD OF STATEMENT

Date: 26th June 2019

Description of Work

To unload 1 unit 7.10.7 CMM machine from lorry chassis using lorry crane, shift and position into tenant premise at ground floor

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

28/06/2019

Time of Operation

Afternoon schedule

Location of Operation

#### Blk 7 Kallang Place #01-06

Lek Hung Moulding Pte Ltd

Equipment Use

1 x lorry crane

Pallet truck

1 set webbing sling

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade operation area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Deploy lorry crane at designated unloading area

- Hoist down machine from lorry chassis

- Lower down machine to near ground level

- Insert pallet truck to machine stand

- Shift and transport machine into tenant premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175